# Research on Robot Manipulator

# Based on Structure of Bi-articular Muscle

Kengo Yoshida, Yoichi Hori

### a Department of Electrical and Electric Engineering, The University of Tokyo,

### 7-3-1 Hongo, Bunkyo, Tokyo, 113-8656, Japan

yoshida@horilab.iis.u-tokyo.ac.jp, hori@iis.u-tokyo.ac.jp

Abstract — This paper describes our research about robot manipulator based on structure of bi-articular muscle. The structure of animal arm is quite different from conventional robot arm. Animal structure contributes their manipulability and control ability. Our research intends to clarify roles of such structure and realize a manipulator equipped with bi-articular driving mechanism. In this paper, Novel Feedforward(FF) path tracking algorithm is described. It utilizes muscular viscoelasticity and it can suppress model error in spite of FF control. Also this algorithm is suitable for robot arm which has bi-articular muscles. Furthermore this paper introduce manufacture of robot arm based on bi-articular muscle principle.

Fig.  1.  Conventional robot arm model and animal's arm model

### Keywords — Robot manipulator, Path tracking, Feedforward algorithm, Bi-articular muscle, Muscular viscoelasticity

### I. Introduction

Today, many robots which have animal-like appearance are researched and developed, and they can imitate animal movements. However they don`t imitate animal's actuators and control mechanisms.

Conventional robots have actuators which drive only one joint, and usually rotational electromagnetic motor are installed in each joint. On the other hand, animals have muscles to move their body. Muscle is a kind of linear actuator which has unique viscoelasticity. They have complex muscular arrangements. Some muscles can drive two or more joints. Fig. 1 shows simplified conventional robot arm model and animal`s arm model.

For control mechanisms, conventional robots need to know precise model of both their body and  environment. Also they need fast feedback control. Animals don't have such good controller. However animals can act more skillfully in uncertain environment. We consider that unique animal arm structures contributes their control ability. Especially muscular viscoelasticity may have a important role instead of fast feedback control.

In this paper, Novel feedforward control algorithm is proposed. The algorithm utilizes muscular viscoelasticity for path tracking and disturbance suppression. We simulate this algorithm and verify its effectiveness. Finally we introduce our experimental robot based on bi-articular muscle principle.

The following are earlier study about bi-articular muscle and muscular viscoelasiticity. Van Ingen Schenau et. al. described a role of bi-articular muscles in vertical jump. Gastrocnemius muscle which is a bi-articular muscles in the calf of the leg develops and transmits propulsive force [1].

Neville Hogan suggested that antagonistic bi-articular muscles can control mechanical impedance. And he showed its effectiveness at contact tasks [2][3]. Mussa-Ivaldi ascertained changes of a stiffness ellipse at the end point of human arms by changing arm postures through experiments [4].

Kumamoto and Oshima et. al. suggested modeling of human arms and legs using two antagonistic pairs of mono-articular muscles and one antagonistic pair of bi-articular muscles. And they revealed that this model can explains recorded EMG patterns when human arms and legs output forces [5][6]. Authors proposed novel design of robot arm equipped with bi-articular muscle [7] and FF path tracking algorithm utilizing muscular viscoelasticity [8].

### II. Model of Viscoelastic Muscle and Muscular Arrangement

Muscle has unique viscoelasticity. Viscoelastic Muscular model is shown in Fig. 2. Muscular output force F is a function of contractile force u.

(1)

F=u−K(u)x−B(u)x=u−kux−bu {x

Here x is contracting length of the muscle and x is shortening velocity. k is elastic coefficient and b is viscosity coefficient. Contractile force u is only settled actively and others are passive elements. In other word u is assumed as activation level of muscle.

Muscles only generate forces when they shrink. Therefore muscles construct antagonistic pair to generate dual-directional force. We think summation and difference of contractile force for antagonistic pair.

Fig. 3. Two Joint Link Model with Both Mono-articular and Bi-articular Muscles

(2)

Mx=D−kSx−bS {x

Eq. (2) is dynamic equation of one-dimensional motion.

D and S are difference and summation of contractile forces. M is mass of object. Summation of contractile forces controls elasticity and viscosity. Output Force is generated by Difference of contractile forces and above passive forces.

In this paper we use simplified two joint link model shown in Fig. 3. In Fig. 3, e1 and f1 are a pair of antagonistic mono-articular muscles attached to joint R1. e2 and f2 are attached to R2. e3 and f3 are a pair of antagonistic bi-articular muscles attached both R1 and R2.

We define output forces of each muscle asFf1 Fe1,Ff2,Fe2,Ff3,Fe3. r1 and r2are radii of R1 and R2. Joint moments T1 and T2  are as follows:

T1=(Ff1+Fe1)r1+(Ff3+Fe3)r1

(3)

T2=(Ff2+Fe2)r2+(Ff3+Fe3)r2

The link model equipped both mono-articular muscles and bi-articular muscles like Fig. 3 can  control output force and stiffness independently at its end point. When summations and differences of contractile forces are defined in each pair of antagonistic muscles, summations control stiffness and differences control force direction at end point. uf1,ue1,uf2,ue2,uf3,ue3 are defined as contractile force in each muscle. Summations S1,S2,S3 and differences D1,D2,D3 are as follows.

S1=uf1+ue1,D1=uf1+ue1

S2=uf2+ue2,D2=uf2+ue2

S3=uf3+ue3,D3=uf3+ue3

(4)

Under the following conditions:

S1>D1,S2>D2,S3>D3

### III. FF Control Algorithm Emulating Muscular Viscoelasticity

When the contractile force of each muscle is settled, an equilibrium position is specified. Even robot arm takes any postures, it goes back to the equilibrium position. To change each contractile force we can drive the robot arm along the path of equilibrium position. At equilibrium joint angles θ1,θ2 are obtained from Eq. (5) in conditions of T1=T2=0 and r1=r2=0.

θ1=1kr(D1+D3)S2+(D1−D2)S3S1S2+S2S3+S3S1

Fig. 2. Model of a Muscle

θ2=1kr(D2+D3)S1+(D2−D1)S3S1S2+S2S3+S3S1

(5)

When expected path of the end point are given, we extract some intermediate point Xm=[xm,ym]T from the path. The subscript m denotes index of intermediate points. We derive desired joint angles θn,m from intermediate points by Eq. (6). The subscript n denotes index of antagonistic pairs.

θ1,m=arctan(ym,xm)−arctan(xm2+ym2−zm2,Zm2)

θ2,m=arctan(ym−l1sinθ1,m,xm−l1cosθ1,m)−θ1,m

θ3,m=θ1,m+θ2,m

(6)

where:

zm=xm2+ym2+l12−l222l1

Eq. (7) is dynamic equation about each θn,m. In is inertia around each joint.

(7)

In(θn,m)=rDn,m−kr2θn,mSn,m−br2θn,mDn,m

In Eq. (8), we decide Sn,m in order that this system become critical damping.

(8)

Sn,m=4kInb2r2

In this case, each joint become constant after settling time. Eq. (9) decides Dn,m by this condition.

(9)

Dn,m=krSn,mθn,m

Tn,m is time length to force by each antagonistic pair. In Eq. (10),  α is coefficient to decide smoothing.

Tn,m=αInbr2Sn,m

(10)

When αis settled 1, Tn,m equals rising time of this system. If intermediate points are settled enough densely,

the arm can track the path smoothly.

IV. Simulation Result Of Proposed Method

Fig. 4. Simulation Model

Simulation model is shown in Fig. 4. Each link is a thin rod which has no thickness and width. Here l1 and l2 are lengths of L1 and L2, m1 and ms are the masses, I1 and I2 are the inertia moments of links related to R1 and R2 , lg1 and lg2 are distances from each center of R1and R2 to each gravity center of L1 and L2. Simulation is calculated using Lagrangian method. Gravitational effect are ignorable because robot arm moves only in horizontal plane.

Simulation result is shown in Fig. 5 and parameters of arm model are shown in TABLE I. The exepected path is shown in Eq. (11).

x−0.30.62+y−0.60.22=1

(11)

We extract 40 intermediate point from this path and decide α as 1.0. This simulation shows that the arm can track expecting path smoothly.

Next we represent robustness of this algorithm. We inputs contractile forces equals to input forces of first simulation (Fig. 5). We assume that m1 is 2.5 kg and ms is 1.0 kg. In this simulation, we change actual masses of each links and its result is shown in Fig. 6. Obviously the result shows amazing robustness of proposed feedforward method. When actual mass is twice as nominal mass, the arm can track expected path (central graph Fig. 6). This robustness is obtained by muscular viscoelasticity. When end point is displaced from expected path, muscular elasticity generates force to suppress its departure. Muscular viscocity stabilize its oscillation.

### V. Experimental Machine

We made robot arm based on bi-articular muscles principle. Fig. 7 is overview of this robot. It has two mono-articular driving mechanisms and one bi-articular driving mechanism. Mono-articular driving mechanism is implementation of antagonistic mono-articular muscle pair. Bi-articular driving mechanism is implementation of antagonistic bi-articular muscle pair.

Fig. 7. Overview of Our Robotarm

Fig. 8. Control Diagram to Generate Force of

Antagonistic Pair

Fig. 9. A Photo of Actual Robot Arm

In this robot, each driving mechanism use electric rotary motors. Muscular viscoelasticity is actualized by a nonlinear feedback control block (Fig. 8). In this block diagram, × means multiplication of inputs.

Each joint angles are detected by rotary encoders to calculated elasticity and viscosity.　Furthermore current bi-articular driving mechanism is incomplete.　We compensate this problem using shoulder side mono-articular driving mechanism.　Fig. 9 is a photo of mechanical part of the robot. TABLE II shows major parameters of robot arm.

V. Conclusion

In this paper we described the new feedforward algorithm for robot arm based on bi-articular muscle principle.This algorithm use muscular viscoelasticity effectively to realize path tracking. The algorithm also has remarkable ability of disturbances suppression in spite of feedforward control. We verified this algorithm by simulation and represent its robustness for model disturbance.

For the experiment we developed novel robot arm. It has bi-articular driving mechanism and emulate muscular viscoelasticity. We will experiment for verification of above algorithm using this robot arm.

TABLE II. Major Parameters of Robt Arm

### References

[1] G. J. van Ingen Shenau and et al., “The unique action of bi-articular muscles in complex move ments”, Journal of Anatomy, 155, pp. 1-5, 1987

[2] Neville Hogan, “Adaptive Control of Mechanical Impedance by Coactivation of Antagonist Muscles”, IEEE Trans. on Automatic Control, vol.AC-29, No.8, pp. 681-690, 1984

[3] Neville Hogan, “On the stability of Manipulators Performing Contact Tasks”, IEEE Journal of Robotics and Automation, Vol. 4, No. 6, pp. 677-686, 1988

[4] F. A. Mussa Ivaldi and et. al., “Neural, Mechanical, and Geometric Factors Subserving Arm Posture in Humans”, The Journal of Neuroscience, Vol. 5, No. 10, pp. 2732-2743, 1985

[5] Mizuyori Kumamoto and et al., “Control properties induced by existence of antagonistic pairs of bi-articular muscles -Mechanical engineering model analyses”, Human Movement Science 13, pp. 611-634, 1994

[6] Toru Oshima and et al., ”Mechanical Properties of Robot Arm Operated with Muscle Coordinate System Consisted of Bi-articular muscles and Monoarticular Muscles - Muscle Contractile Forces and Viscoelastic Properties of Robot Arm -”, Journal of The Japan Society for Precision Engineering, vol. 66, No. 1, pp. 141-146, 2000.

[7] Kengo Yoshida and et al., “A Novel Design and Realization of Robot Arm Based on the Principle of Bi-articular Muscles”, Proc. IEEE ICIT, 2006

[8] Kengo Yoshida and et al., "Novel FF Control Algorithm of Robot Arm Based on Bi-articular Muscle Principle - Emulation of Muscular Viscoelasticity for Disturbance Suppression and Path Tracking", Proc. IEEE IECON, 2007

Fig, 5. Simulation Result of Driving Robot Arm Using Muscular Viscoelasticity.

Left image is trajectory of endpoint. Center image is change of joint angles. Contractile forces  are indicated in right image

.

Fig. 6. Simulation Result of Changing Mass of Each Link